Industrial I/O and Customized Connectors and Cable Assembly

- Industrial Ethernet now links automation technology to standard networks and the Internet world.

- We are proud to lead the way in standard connectors linking the office world to the demanding industrial setting and offer special solutions for cabling and assembling.

- Our I/0 connectors are sealed to an IP67 rating, along with EMC protection. We offer various products, including M12, M8, RJ45, RJ12, USB 2.0 / USB 3.0, Din, Mini-DIN, D-sub, Optical Fiber and customized connector and cable assembly

- TPU over molded or PA+GF housing

- Copper alloy contacts and shell

- Cable length customized, PUR and PVT cable options

- The fire resistance grade of raw cable is optional as below

  - CMP/CMR/CM/CMG/CMX/ VW-1/VW-2/FT-1/FT-2/UL 94 V-0/V-1/V-2/V-3

- Water resistance is optional with IP65/67/68

- Cable bending test is optional with 30℃, 45℃, 90℃

Content Table of Connector and Cable Assembly

| - FAKRA and Mini FAKRA Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| FAKRA female Coding A-Z cable assembly | 5-8 | High Data Rates Up to 8Gbps |
| FAKRA male Coding A-Z cable assembly | 9-12 | High Data Rates Up to 8Gbps |
| Mini FAKRA female Coding A-Z cable assembly | 13-16 | High Data Rates Up to 20Gbps |

| - Energy Storage/Solar Energy/Photovoltaic Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| Energy Storage: 2+12 Pin Straight, Molded Cable Assembly | 17-18 |  |
| MC4 Female Straight, Cable Assembly | 19-20 |  |
| MC4 male Straight, Cable Assembly | 21-22 |  |
| M23 male straight molded cable assembly for charging/discharging battery | 23-24 |  |
| M23 male with base cable assembly for charging/discharging battery | 25-26 |  |
| M23 Female with base, cable assembly for charging/discharging battery | 27-28 |  |

| - Customized Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| USB Type C Female, IP68, Cable Assembly (Customized) | 29-30 |  |
| 4P Male IP67 for start system, Cable Assembly (Customized) | 31-32 |  |
| 4P female IP67 for start system Cable Assembly (Customized) | 33-34 |  |

| - CPC (Plastic) Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| CPC Female (Series1-6) Cable Assembly | 35-38 |  |
| CPC male (Series 1-6) Cable Assembly | 39-42 |  |

| - M12 Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| M12, A code, female, straight molded, cable assembly | 43-44 | A-coded for sensors, dc power and 1 Gbit Ethernet |
| M12, A code, male, straight molded, cable assembly | 45-46 |  |
| M12, A code, female, right angle molded, cable assembly | 47-48 |  |
| M12, A code, male, right angle molded, cable assembly | 49-50 |  |
| M12, D code, male, straight molded, cable assembly | 51-52 | D-coded for 100 Mbit Ethernet |
| M12, D code, female, straight molded, cable assembly | 53-54 |  |
| M12, X code male, straight molded, cable assembly | 55-56 | X-coded for 10 Gbit Ethernet |

| - M8 Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| M8, A code, female straight molded, cable assembly | 57-58 | Right angle can be customized |
| M8, A code, male straight molded, cable assembly | 59-60 | Right angle can be customized |

| - DIN and MINI DIN Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| DIN Female straight molded, cable assembly | 61-62 | 3-8 Pins options; right angle can be customized |
| DIN Male straight molded, cable assembly | 63-64 | 3-8 Pins options |
| DIN Male right angle molded, cable assembly | 65-66 | 3-8 Pins options |
| Mini DIN female, straight molded, cable assembly | 67-68 | 3-9 Pins options |
| Mini DIN male, straight molded, cable assembly | 69-70 | 3-9 Pins options |

| - D-SUB (Standard) Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| 9 Pin D-sub female, straight molded, cable assembly | 71-72 | Right angle can be customized |
| 9 Pin D-sub male, straight molded, cable assembly | 73-74 | Right angle can be customized |
| 15 Pin D-sub female, straight molded, cable assembly | 75-76 | Right angle can be customized |
| 15 Pin D-sub male, straight molded, cable assembly | 77-78 | Right angle can be customized |
| 25 Pin D-sub female, straight molded, cable assembly | 79-80 | Right angle can be customized |
| 25 Pin D-sub male, straight molded, cable assembly | 81-82 | Right angle can be customized |
| 37 Pin D-sub female, straight molded, cable assembly | 83-84 | 50Pins options, Right angle can be customized |
| 37 Pin D-sub male, straight molded, cable assembly | 85-86 | 50Pins options, Right angle can be customized |

| - HDB D-SUB, Hybrid D-SUB and Micro D-SUB Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| 15 Pin HDB D-sub female, straight molded, cable assembly | 87-88 | 15, 26, 44, 62, 78 Pins options |
| 15 Pin HDB D-sub male, straight molded, cable assembly | 89-90 | 15, 26, 44, 62, 78 Pins options |
| 27W2 Hybrid D-SUB female molded cable assembly, straight | 91-92 | 27W2, 7W2, 11W1, 13W3, 9W4, 5W5 pins options |
| 27W2 Hybrid D-SUB male molded cable assembly, straight | 93-94 | 27W2, 7W2, 11W1, 13W3, 9W4, 5W5 pins options |
| 9Pin Micro D-SUB Female molded cable assembly, straight | 95-96 | 9, 15, 25 pins options |

| - USB2.0 (480Mbps(60MB/s)) Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| USB2.0 Type A Female, straight molded cable assembly | 97-98 |  |
| USB2.0 Type A male, straight molded cable assembly | 99-100 |  |
| USB2.0 Type A male, right angle molded cable assembly | 101-102 |  |
| USB2.0 Type B male, straight molded cable assembly | 103-104 |  |
| USB2.0 Type B male, Right angle molded cable assembly | 105-106 |  |
| USB2.0 Mini B male, Straight molded cable assembly | 107-108 |  |
| USB2.0 Micro B male, Straight molded cable assembly | 109-110 |  |
| USB2.0 Micro B male, Right angle molded cable assembly | 111-112 |  |

| - USB3.0 (5Gbps(640MB/s)) Cable Assembly |  |  |
| --- | --- | --- |
| Products | Pages | Remark |
| USB3.0 Type A Female, Straight molded cable assembly | 113-114 |  |
| USB3.0 Type A male, Straight molded cable assembly | 115-116 |  |
| USB3.0 Micro B male, Straight molded cable assembly | 117-118 |  |
| Type C male, Straight molded cable assembly | 119-120 |  |

FAKRA Female Connector with Cable Assembly (A Coding to Z Coding)

Product Description

FAKRA (Automated Expert Group) to establish a unified connector system and standards that everyone will follow. Due to its special standard locking system, FAKRA connectors meet the high performance and safety requirements of today's automotive industry.

- General Information of Cable Assembly

| Feature/Speed | -Primary and secondary locking systems for increased assembly security -13 mechanical and colored coding with one neutral coding, refer to FAKRA coding on next page -20 Gbps of data speed for the connected vehicle, supporting any modern radar, camera, lidar or sensor applications |
| --- | --- |
| Rated Current | 1A Max |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -40℃ ~+105℃ |
| Poles | 1,2,4 Poles are available（Refer to SKU table） |
| Cable | RG 174, RTK 031, RG59, RG179, RG58/62 etc. (50, 61,75, 93,120 Ohm) options |
| Rentention Force Latch | ≥ 110 N |
| Rentention Force Lock | ≥80N (Primary) ≥60N (Secondary) |
| Compliant to Standard | Per ISO20860-1 and -2, USCAR17 and 18 |
| Water Resistance Level | No/IPX7/IPX9K in locked condition options |
| Application | Driving modules, Camera, Antenna, GPS, Gateway/Switch modules, 4K displays, High-speed cable networking, Sensor-device connectivity, surround view cameras, Infotainment devices |

Dimensions of Related Components (Unit is mm)

- General Information of Connector

Frequency Range:              DC-6GHz               Insulation Resistance:   1000MΩ min.

Working Voltage:               335V max.             Impedance:     50 Ω Max.

Center Contact Resistance:        20mΩ max.           Return Loss:     ≥ 18 dB

Dielectric Withstanding Voltage:   1000V min.            IP Rating:     No/IPX7/IPX9K in locked condition (options)

Temperature Range:            -40 °C to +105 °C        Housing/Center contact: PBT, PA/CuZn, CuBe, CuSn

V.S.W.R(电压驻波比):           1.3 Max.               Mating Endurance:      ≥25 Cycles

High Data Rates:                Up to 8Gbps

- SKU Table

| SKU No. | Poles | Conn. Type | Type | Remark |
| --- | --- | --- | --- | --- |
| T-FAKRASTF-01-Coding A-Z | 1 | Female | Straight | Coding A-Z |
| T-FAKRASTF-02-D8-Coding A-Z | 2 | Female | Straight | Double 8mm type, Coding A-Z |
| T-FAKRASTF-02-D12.7-Coding A-Z | 2 | Female | Straight | Double 12.7mm type, Coding A-Z |
| T-FAKRASTF-04-Coding A-Z | 4 | Female | Straight | Coding A-Z |

Notes: Coding A-Z color

A Black

B White

C Blue

D Bordeaux violet

E Green

F Brown

G Grey

H Violet

I Beige

K Curry

L Carmin red

M Pastel orange

N Pastel green

Z Water blue

FAKRA Male Connector Cable Assembly, (A Coding to Z Coding)

Product Description

FAKRA (Automated Expert Group) to establish a unified connector system and standards that everyone will follow. Due to its special standard locking system, FAKRA connectors meet the high performance and safety requirements of today's automotive industry.

- General Information of Cable Assembly

| Feature/Speed | -Primary and secondary locking systems for increased assembly security -13 mechanical and colored coding with one neutral coding, refer to FAKRA coding on next page -20 Gbps of data speed for the connected vehicle, supporting any modern radar, camera, lidar or sensor applications |
| --- | --- |
| Rated Current | 1A Max |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -40℃ ~+105℃ |
| Poles | 1,2 Poles are available（Refer to SKU table） |
| Cable | RG 174, RTK 031, RG59, RG179, RG58/62 etc. (50, 61,75, 93,120 Ohm) options |
| Rentention Force Latch | ≥ 110 N |
| Rentention Force Lock | ≥80N (Primary) ≥60N (Secondary) |
| Compliant to Standard | Per ISO20860-1 and -2, USCAR17 and 18 |
| Water Resistance Level | No/IPX7/IPX9K in locked condition options |
| Application | Driving modules, Camera, Antenna, GPS, Gateway/Switch modules, 4K displays, High-speed cable networking, Sensor-device connectivity, surround view cameras, Infotainment devices |

Dimensions of Related Components (Unit is mm)

- General Information of Connector

Frequency Range:               DC-6GHz                  Insulation Resistance:   1000MΩ min.

Working Voltage:               335V max.                Impedance:     50 Ω Max.

Center Contact Resistance:        20mΩ max.              Return Loss:     ≥ 18 dB

Dielectric Withstanding Voltage:   1000V min.               IP Rating:     IPX7/IPX9K in locked condition (options)

Temperature Range:            -40 °C to +105 °C           Housing/Center contact: PBT, PA/CuZn, CuBe, CuSn

V.S.W.R(电压驻波比):           1.3 Max.                 Mating Endurance:      ≥25 Cycles

High Data Rates: Up to 8Gbps

- SKU Table

| SKU No. | Poles | Conn. Type | Type | Remark |
| --- | --- | --- | --- | --- |
| T-FAKRASTM-01-Coding A-Z | 1 | Male | Straight | Coding A-Z |
| T-FAKRASTM-02-D8-Coding A-Z | 2 | Male | Straight | Double 8mm type, Coding A-Z |
| T-FAKRASTM-02-D12.7-Coding A-Z | 2 | Male | Straight | Double 12.7mm type, Coding A-Z |

Notes: Coding A-Z color

A Black

B White

C Blue

D Bordeaux violet

E Green

F Brown

G Grey

H Violet

I Beige

K Curry

L Carmin red

M Pastel orange

N Pastel green

Z Water blue

MINI FAKRA Female Connector with Cable Assembly

Product Description

The High-Speed FAKRA Mini Coaxial Cable Solution delivers 20 Gbps of data speed for the connected vehicle, reducing installation space requirements by up to 80 percent compared to traditional FAKRA, supporting any modern radar, camera, lidar or sensor applications.

- General Information of Cable Assembly

| Feature/Speed | -Primary and secondary locking systems for increased assembly security -Multiple color-coded enclosures provide visual guidance during assembly and prevent connectors from mismatching with different keyway options. -20 Gbps of data speed for the connected vehicle, supporting any modern radar, camera, lidar or sensor applications |
| --- | --- |
| Rated Current | 1A Max |
| Rated Voltage | 60V AC/DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -40℃ ~+105℃ |
| Poles | 1,2,4 Poles are available（Refer to SKU table） |
| Cable | RG 174, RTK 031, RTK044 etc. (50/75 Ohm) options |
| Rentention Force Latch | ≥ 110 N |
| Rentention Force Lock | ≥80N (Primary) ≥60N (Secondary) |

Dimensions of Related Components (Unit is mm)

- General Information of Connector

Frequency Range:               DC-15GHz              Insulation Resistance:   1000MΩ min.

Working Voltage:               335V max.              Impedance:            50 Ω Max.

Center Contact Resistance:        15mΩ max.             High Data Rates:       Up to 8Gbps

Dielectric Withstanding Voltage:   1000V min.             Mating Endurance:      ≥25 Cycles

Temperature Range:            -40 °C to +105 °C          Housing/Center contact: PBT-GF

Return Loss:  ≥ 25 dB@DC-3GHz, (≥ -20db@3-6GHz, ≥ -12db@6-9GHz)

- SKU Table

| SKU No. | Poles | Conn. Type | Type | Remark |
| --- | --- | --- | --- | --- |
| T-MFAKRASTF-01-Coding A-Z | 1 | Female | Straight | Coding A-Z |
| T-MFAKRASTF-02-Coding A-Z | 2 | Female | Straight | Coding A-Z |
| T-MFAKRASTF-04-Coding A-Z | 4 | Female | Straight | Coding A-Z |

Notes: Coding A-Z color

A Black

B White

C Blue

D Bordeaux violet

E Green

F Brown

G Grey

H Violet

I Beige

K Curry

L Carmin red

M Pastel orange

N Pastel green

Z Water blue

Energy Storage: 2+12 Pin Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 100A |
| --- | --- |
| Insulation Resistance | ≥100MΩ |
| Operation Temperature | -40℃ ~+80℃ |
| Poles | 2+12 Pin （Refer to below SKU table） |
| Water Resistance Level | IP30 in locked condition |
| Cable | UL2517 2*5AWG+10*22AWG VW-1/FT1, PVC cable |
| Temperature Rise Test | 120A current lasted for 4H, temperature rise<50K (5AWG/17mm) |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-ESSTF-14-001 | 2+12 | Female | Straight |  |

Dimensions of Mating Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -40℃ ~ +80℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PA66+GF/94V-0                  Insert/Pull Force:       <100N

Connector Contacts:     Brass with silver plated            IP Rating:             IP30 in locked condition

Compression Strength:   DC500V, leakage current <5mA     Mating Endurance:      3000 Cycles

Solar Energy/Photovoltaic: MC4 Female Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 30A |
| --- | --- |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | <0.35mΩ |
| Operation Temperature | -40℃ ~+80℃ |
| Poles | 1 Pin （Refer to below SKU table） |
| Water Resistance Level | IP67/IP68 in locked condition, optional |
| Cable | 16-20AWG VW-1/FT1, PVT or customized |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-MC4STF-01-001 | 1 | Female | Straight |  |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -40℃ ~ +80℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PA66+GF/94V-0                  Insert/Pull Force:       <100N

Connector Contacts:     Brass with silver plated            IP Rating:       IP67/IP68 in locked condition optional

Rated Current/Voltage:   30A/1000V

Solar Energy/Photovoltaic: MC4 Male Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 30A |
| --- | --- |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | <0.35mΩ |
| Operation Temperature | -40℃ ~+80℃ |
| Poles | 1 Pin （Refer to below SKU table） |
| Water Resistance Level | IP67/IP68 in locked condition, optional |
| Cable | 16-20AWG VW-1/FT1, PVT or customized |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-MC4STF-01-001 | 1 | Male | Straight |  |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -40℃ ~ +80℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PA66+GF/94V-0                  Insert/Pull Force:       <100N

Connector Contacts:     Brass with silver plated            IP Rating:       IP67/IP68 in locked condition optional

Rated Current/Voltage:   30A/1000V

M23 Male Charging/Discharging Battery Connector, Molded Cable Assembly

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 50A/10A/3A |
| --- | --- |
| Rated Voltage | 500V/300V/60V /DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤10 mΩ |
| Operation Temperature | -40℃ ~+105℃ |
| Poles | 2+0, 2+5, 2+1+3, 2+1+5 Pins Optional (Refer to below SKU table and pin picture）, straight or right angle optional |
| Water Resistance Level | IP67/IP68 in locked condition, optional |
| Cable | 8AWG*2C+16AWG*1C+22AWG*5C,PVC or customized |

- Difference Pins as Below:

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-EM23STM-2-001 | 2+0 | Male | Straight |  |
| T-EM23STM-7-001 | 2+5 | Male | Straight |  |
| T-EM23STM-6-001 | 2+1+3 | Male | Straight |  |
| T-EM23STM-8-001 | 2+1+5 | Male | Straight |  |
| T-EM23RAM-2-001 | 2+0 | Male | Right Angle |  |
| T-EM23RAM-7-001 | 2+5 | Male | Right Angle |  |
| T-EM23RAM-6-001 | 2+1+3 | Male | Right Angle |  |
| T-EM23RAM-8-001 | 2+1+5 | Male | Right Angle |  |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -40℃ ~ +105℃                 Insulation Resistance:   ≥100MΩ

Connector Body:        UL94V-0                        Rated Current:       20-50A

Connector Contacts:     Brass with silver/golded plated    IP Rating:    IP67/IP68 in locked condition optional

M23 Male With Base, Charging/Discharging Battery Connector, Cable Assembly

- General Information of Cable Assembly

| Rated Current | 50A/10A/3A |
| --- | --- |
| Rated Voltage | 500V/300V/60V /DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤10 mΩ |
| Operation Temperature | -40℃ ~+105℃ |
| Poles | 2+1+5 Pins is available (Refer to below SKU table), 2+0, 2+5, 2+1+3 PIN can be customized |
| Water Resistance Level | IP67/IP68 in locked condition, optional |
| Cable | 8AWG*2C+16AWG*1C+22AWG*5C,PVC or customized |

- SKU Table

| SKU No. | Poles | Conn. Type | Type | Remark |
| --- | --- | --- | --- | --- |
| T-EM23BM-2-001 | 2+0 | Male | Assembly |  |
| T-EM23BM-7-001 | 2+5 | Male | Assembly |  |
| T-EM23BM-6-001 | 2+1+3 | Male | Assembly |  |
| T-EM23BM-8-001 | 2+1+5 | Male | Assembly |  |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -40℃ ~ +105℃                 Insulation Resistance:   ≥100MΩ

Connector Body:        UL94V-0                        Rated Current:       20-50A

Connector Contacts:     Brass with silver/golded plated    IP Rating:    IP67/IP68 in locked condition optional

M23 Female Charging/Discharging Battery Connector, Cable Assembly

- General Information of Cable Assembly

| Rated Current | 50A/10A/3A |
| --- | --- |
| Rated Voltage | 500V/300V/60V /DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤10 mΩ |
| Operation Temperature | -40℃ ~+105℃ |
| Poles | 2+0, 2+5, 2+1+3, 2+1+5 Pins Optional (Refer to below SKU table and pin picture） |
| Water Resistance Level | IP67/IP68 in locked condition, optional |
| Cable | 8AWG*2C+16AWG*1C+22AWG*5C,PVC or customized |

- Difference Pins as Below:

- SKU Table

| SKU No. | Poles | Conn. Type | Type | Remark |
| --- | --- | --- | --- | --- |
| T-EM23BF-2-001 | 2+0 | Female | Assembly |  |
| T-EM23BF-7-001 | 2+5 | Female | Assembly |  |
| T-EM23BF-6-001 | 2+1+3 | Female | Assembly |  |
| T-EM23BF-8-001 | 2+1+5 | Female | Assembly |  |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -40℃ ~ +105℃                 Insulation Resistance:   ≥100MΩ

Connector Body:        UL94V-0                        Rated Current:       20-50A

Connector Contacts:     Brass with silver/golden plated      IP Rating:    IP67/IP20/IP68 in locked condition optional

Customized, Type C, Female Cable Assembly, IP68

- General Information of Cable Assembly

| Rated Voltage | 30V /DC |
| --- | --- |
| Insulation Resistance | ≥5MΩ |
| Contact Resistance | ≤3Ω |
| Water Resistance Level | IP68 in locked condition |
| Cable | UL2464 22AWG*4C PVC blk, can be customized |

Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Type | Remark |
| --- | --- | --- | --- | --- |
| T-CUSTC4F-002 | 4 | Female | Assembly | Customized |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +85℃                  Insulation Resistance:   ≥5MΩ

Connector Body:        PBT, black                      Fire resistance：        UL94V-0

Connector Contacts:     Brass with silver/golden plated

Customized, 4Pin Male Cable Assembly For Smart System, IP67

Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Voltage | 12V /DC |
| --- | --- |
| Insulation Resistance | ≥10MΩ |
| Contact Resistance | ≤3Ω |
| Water Resistance Level | IP67 in locked condition |
| Cable | Cable, length can be customized |
| Application | - Original Bosch replacement cable, Connects the drive unit with the control unit |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmold Type | Remark |
| --- | --- | --- | --- | --- |
| T-CUS4M-002 | 4 | Male | Straight | Customized |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PA66, black                     Fire resistance：       UL94V-0

Connector Contacts:     Brass with golden plated          Water Resistance:      IP66

Customized, 4Pin Female Cable Assembly For Smart System, IP67

Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Voltage | 12V /DC |
| --- | --- |
| Insulation Resistance | ≥10MΩ |
| Contact Resistance | ≤3Ω |
| Water Resistance Level | IP67 in locked condition |
| Cable | Cable, length can be customized |
| Application | - Original Bosch replacement cable, Connects the drive unit with the control unit |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmold Type | Remark |
| --- | --- | --- | --- | --- |
| T-CUS4F-002 | 4 | Female | Straight | Customized |

Dimensions of Related Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PA66, black                     Fire resistance：       UL94V-0

Connector Contacts:     Brass with golden plated           Water Resistance:      IP66                   vel

CPC 9PIN Female Molded Cable Assembly

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 13A Max.(Series1); 7.5A Max.(Series2); 25A Max.(Series3); 25A Max.(Series4); 50A Max.(Series5); 50A Max.(Series6); |
| --- | --- |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 8,17 Poles are available, 3,4,5,12 can be customized （Refer to below SKU table） |
| Water Resistance Level | IP67, IP68 can be customized (In locked condition) |
| Cable | PUR/PVC cable options |

- Shape A and Shape B of Locked Key （Optional）

- Pinouts（Optional for difference CNC connector）

- SKU Table

| Assembly SKU No. | Connector Information |  |  |  |  | Cable Assembly |
| --- | --- | --- | --- | --- | --- | --- |
|  | Poles | Conn. Type | Locked Key | Series | Shell Size | Overmoding or Assembly Type |
| T-CPCS1STF-5-04-001-Shell 11 | 4 | Female | 5 | 1 | 11 | Customized |
| T-CPCS1STF-4-04-001-Shell 11 | 4 | Female | 4 | 1 | 11 | Customized |
| T-CPCS1STF-5-09-001-Shell 13 | 9 | Female | 5 | 1 | 13 | Optional |
| T-CPCS1STF-4-09-001-Shell 13 | 9 | Female | 4 | 1 | 13 | Optional |
| T-CPCS1STF-5-14-001-Shell 17 | 14 | Female | 5 | 1 | 17 | Customized |
| T-CPCS1STF-4-14-001-Shell 17 | 14 | Female | 4 | 1 | 17 | Customized |
| T-CPCS1STF-5-16-001-Shell 17 | 16 | Female | 5 | 1 | 17 | Customized |
| T-CPCS1STF-4-16-001-Shell 17 | 16 | Female | 4 | 1 | 17 | Customized |
| T-CPCS1STF-5-24-001-Shell 23 | 24 | Female | 5 | 1 | 23 | Customized |
| T-CPCS1STF-4-24-001-Shell 23 | 24 | Female | 4 | 1 | 23 | Customized |
| T-CPCS1STF-5-37-001-Shell 23 | 37 | Female | 5 | 1 | 23 | Customized |
| T-CPCS1STF-4-37-001-Shell 23 | 37 | Female | 4 | 1 | 23 | Customized |

| Assembly SKU No. | Connector Information |  |  |  |  | Cable Assembly |
| --- | --- | --- | --- | --- | --- | --- |
|  | Poles | Conn. Type | Locked Key | Series | Shell Size | Overmoding or Assembly Type |
| T-CPCS2STF-5-08-001-Shell 11 | 8 | Female | 5 | 2 | 11 | Customized |
| T-CPCS2STF-4-08-001-Shell 11 | 8 | Female | 4 | 2 | 11 | Customized |
| T-CPCS2STF-5-09-001-Shell 11 | 9 | Female | 5 | 2 | 11 | Optional |
| T-CPCS2STF-4-09-001-Shell 11 | 9 | Female | 4 | 2 | 11 | Optional |
| T-CPCS2STF-5-28-001-Shell 17 | 28 | Female | 5 | 2 | 17 | Customized |
| T-CPCS2STF-4-28-001-Shell 17 | 28 | Female | 4 | 2 | 17 | Customized |
| T-CPCS2STF-5-57-001-Shell 23 | 57 | Female | 5 | 2 | 23 | Customized |
| T-CPCS2STF-4-57-001-Shell 23 | 57 | Female | 4 | 2 | 23 | Customized |
| T-CPCS2STF-5-63-001-Shell 23 | 63 | Female | 5 | 2 | 23 | Customized |
| T-CPCS2STF-4-63-001-Shell 23 | 63 | Female | 4 | 2 | 23 | Customized |
| T-CPCS3STF-5-3-001-Shell 17 | 3 | Female | 5 | 3 | 17 | Customized |
| T-CPCS3STF-4-3-001-Shell 17 | 3 | Female | 4 | 3 | 17 | Customized |
| T-CPCS3STF-5-7-001-Shell 23 | 7 | Female | 5 | 3 | 23 | Customized |
| T-CPCS3STF-4-7-001-Shell 23 | 7 | Female | 4 | 3 | 23 | Customized |
| T-CPCS4STF-5-13-001-Shell 23 | 13 | Female | 5 | 4 | 23 | Customized |
| T-CPCS4STF-4-13-001-Shell 23 | 13 | Female | 4 | 4 | 23 | Customized |
| T-CPCS4STF-5-16-001-Shell 23 | 16 | Female | 5 | 4 | 23 | Customized |
| T-CPCS4STF-4-16-001-Shell 23 | 16 | Female | 4 | 4 | 23 | Customized |
| T-CPCS4STF-5-22-001-Shell 23 | 22 | Female | 5 | 4 | 23 | Customized |
| T-CPCS4STF-4-22-001-Shell 23 | 22 | Female | 4 | 4 | 23 | Customized |
| T-CPCS5STF-5-3-001-Shell 17 | 3 | Female | 5 | 5 | 17 | Customized |
| T-CPCS5STF-4-3-001-Shell 17 | 3 | Female | 4 | 5 | 17 | Customized |
| T-CPCS6STF-5-16-001-Shell 23 | 16 | Female | 5 | 6 | 23 | Customized |
| T-CPCS6STF-4-16-001-Shell 23 | 16 | Female | 4 | 6 | 23 | Customized |

Dimensions of Related Components (Unit is inch)

- General Information of Connector

Ambient Temperature:   -55℃ ~ +105℃                 Insulation Resistance:   ≥100MΩ

Connector Body:        Glass filled nylon 6/6,black         IP Rating:             IP65,IP67 optional

Connector Contacts:     Brass with gold plated

- CPC Series Information of Connector

| Items | Description | Contact Shape |
| --- | --- | --- |
| Series 1 | Standard, Terminal OD with 1.57mm, 13A Max, 4-37P |  |
| Series 2 | High Density, Terminal OD with 1.02mm, 7.5A Max, 8-63P |  |
| Series 3 | Power Density, 25A Max., 13P, 16P, 22P |  |
| Series 4 | Power and Signal combine, Signal 13A max.; Power 25A Max., 13P, 16P, 22P |  |
| Series 5 | Power Density (Seal IP65/67 optional), Power with 50A Max., with 3x Powerband |  |
| Series 6 | Power and Signal combine (Seal optional), Signal 13A max., Power 50A Max., with 2x powerband and 8x III+ signal contact |  |

CPC 9PIN Male Molded Cable Assembly

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 13A Max.(Series1); 7.5A Max.(Series2); 25A Max.(Series3); 25A Max.(Series4); 50A Max.(Series5); 50A Max.(Series6); |
| --- | --- |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 8,17 Poles are available, 3,4,5,12 can be customized （Refer to below SKU table） |
| Water Resistance Level | IP67, IP68 can be customized ( in locked condition) |
| Cable | PUR/PVC cable options |

- Shape A and Shape B of Locked Key （Optional）

- Pinouts（Optional for difference CNC connector）

- SKU Table

| Assembly SKU No. | Connector Information |  |  |  |  | Cable Assembly |
| --- | --- | --- | --- | --- | --- | --- |
|  | Poles | Conn. Type | Locked Key | Series | Shell Size | Overmoding or Assembly Type |
| T-CPCS1STM-5-04-001-Shell 11 | 4 | Male | 5 | 1 | 11 | Customized |
| T-CPCS1STM-4-04-001-Shell 11 | 4 | Male | 4 | 1 | 11 | Customized |
| T-CPCS1STM-5-09-001-Shell 13 | 9 | Male | 5 | 1 | 13 | Optional |
| T-CPCS1STM-4-09-001-Shell 13 | 9 | Male | 4 | 1 | 13 | Optional |
| T-CPCS1STM-5-14-001-Shell 17 | 14 | Male | 5 | 1 | 17 | Customized |
| T-CPCS1STM-4-14-001-Shell 17 | 14 | Male | 4 | 1 | 17 | Customized |
| T-CPCS1STM-5-16-001-Shell 17 | 16 | Male | 5 | 1 | 17 | Customized |
| T-CPCS1STM-4-16-001-Shell 17 | 16 | Male | 4 | 1 | 17 | Customized |
| T-CPCS1STM-5-24-001-Shell 23 | 24 | Male | 5 | 1 | 23 | Customized |
| T-CPCS1STM-4-24-001-Shell 23 | 24 | Male | 4 | 1 | 23 | Customized |
| T-CPCS1STM-5-37-001-Shell 23 | 37 | Male | 5 | 1 | 23 | Customized |
| T-CPCS1STM-4-37-001-Shell 23 | 37 | Male | 4 | 1 | 23 | Customized |

| Assembly SKU No. | Connector Information |  |  |  |  | Cable Assembly |
| --- | --- | --- | --- | --- | --- | --- |
|  | Poles | Conn. Type | Locked Key | Series | Shell Size | Overmoding or Assembly Type |
| T-CPCS2STM-5-08-001-Shell 11 | 8 | Male | 5 | 2 | 11 | Customized |
| T-CPCS2STM-4-08-001-Shell 11 | 8 | Male | 4 | 2 | 11 | Customized |
| T-CPCS2STM-5-09-001-Shell 11 | 9 | Male | 5 | 2 | 11 | Optional |
| T-CPCS2STM-4-09-001-Shell 11 | 9 | Male | 4 | 2 | 11 | Optional |
| T-CPCS2STM-5-28-001-Shell 17 | 28 | Male | 5 | 2 | 17 | Customized |
| T-CPCS2STM-4-28-001-Shell 17 | 28 | Male | 4 | 2 | 17 | Customized |
| T-CPCS2STM-5-57-001-Shell 23 | 57 | Male | 5 | 2 | 23 | Customized |
| T-CPCS2STM-4-57-001-Shell 23 | 57 | Male | 4 | 2 | 23 | Customized |
| T-CPCS2STM-5-63-001-Shell 23 | 63 | Male | 5 | 2 | 23 | Customized |
| T-CPCS2STM-4-63-001-Shell 23 | 63 | Male | 4 | 2 | 23 | Customized |
| T-CPCS3STM-5-3-001-Shell 17 | 3 | Male | 5 | 3 | 17 | Customized |
| T-CPCS3STM-4-3-001-Shell 17 | 3 | Male | 4 | 3 | 17 | Customized |
| T-CPCS3STM-5-7-001-Shell 23 | 7 | Male | 5 | 3 | 23 | Customized |
| T-CPCS3STM-4-7-001-Shell 23 | 7 | Male | 4 | 3 | 23 | Customized |
| T-CPCS4STM-5-13-001-Shell 23 | 13 | Male | 5 | 4 | 23 | Customized |
| T-CPCS4STM-4-13-001-Shell 23 | 13 | Male | 4 | 4 | 23 | Customized |
| T-CPCS4STM-5-16-001-Shell 23 | 16 | Male | 5 | 4 | 23 | Customized |
| T-CPCS4STM-4-16-001-Shell 23 | 16 | Male | 4 | 4 | 23 | Customized |
| T-CPCS4STM-5-22-001-Shell 23 | 22 | Male | 5 | 4 | 23 | Customized |
| T-CPCS4STM-4-22-001-Shell 23 | 22 | Male | 4 | 4 | 23 | Customized |
| T-CPCS5STM-5-3-001-Shell 17 | 3 | Male | 5 | 5 | 17 | Customized |
| T-CPCS5STM-4-3-001-Shell 17 | 3 | Male | 4 | 5 | 17 | Customized |
| T-CPCS6STM-5-16-001-Shell 23 | 16 | Male | 5 | 6 | 23 | Customized |
| T-CPCS6STM-4-16-001-Shell 23 | 16 | Male | 4 | 6 | 23 | Customized |

Dimensions of Related Components (Unit is mm/inch)

- General Information of Connector

Ambient Temperature:   -55℃ ~ +105℃                 Insulation Resistance:   ≥100MΩ

Connector Body:        Glass filled nylon 6/6,black         IP Rating:             IP65,IP67 optional

Connector Contacts:     Brass with gold plated

- CPC Series Information of Connector

| Items | Description | Contact Shape |
| --- | --- | --- |
| Series 1 | Standard, Terminal OD with 1.57mm, 13A Max, 4-37P |  |
| Series 2 | High Density, Terminal OD with 1.02mm, 7.5A Max, 8-63P |  |
| Series 3 | Power Density, 25A Max., 13P, 16P, 22P |  |
| Series 4 | Power and Signal combine, Signal 13A max.; Power 25A Max., 13P, 16P, 22P |  |
| Series 5 | Power Density (Seal IP65/67 optional), Power with 50A Max., with 3x Powerband |  |
| Series 6 | Power and Signal combine (Seal optional), Signal 13A max., Power 50A Max., with 2x powerband and 8x III+ signal contact |  |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-M12ASTM-03-001 | 3 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-04-001 | 4 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-05-001 | 5 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-08-001 | 8 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-12-001 | 12 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-17-001 | 17 | Male | Straight | With Metal Nut/Screw thread |

M12 Female (A Coding) Molded Cable, Straight, With Metal Nut (Screw Thread)

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 2A Max (6-8Pins), 4A Max (2-5Pins), 1.5A Max. (9-17Pins) |
| --- | --- |
| Rated Voltage | 30/60V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4, 8,12 Poles are available, 3,5,17 can be customized （Refer to below SKU table） |
| Water Resistance Level | IP67, IP68 in locked condition |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-M12ASTF-03-001 | 3 | Female | Straight | With Metal Nut/Screw thread |
| T-M12ASTF-04-001 | 4 | Female | Straight | With Metal Nut/Screw thread |
| T-M12ASTF-05-001 | 5 | Female | Straight | With Metal Nut/Screw thread |
| T-M12ASTF-08-001 | 8 | Female | Straight | With Metal Nut/Screw thread |
| T-M12ASTF-12-001 | 12 | Female | Straight | With Metal Nut/Screw thread |
| T-M12ASTF-17-001 | 17 | Female | Straight | With Metal Nut/Screw thread |

Dimensions of Related Components  (Unit is mm)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                 Insulation Resistance:   ≥100MΩ

Connector Body:        PA66                         Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with gold plated            Shield:               Available

Coupling Nut/Screw:     Zinc alloy with nickel plated       IP Rating:             IP67,IP68 in locked condition

Seal/O-ring:            Silicone                       Mating Endurance:      >500 Cycles

M12 Male (A Coding) Molded Cable, Straight, With Metal Nut (Screw Thread)

- Dimensions of Cable Assembly  (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 2A Max (6-8Pins), 4A Max (2-5Pins), 1.5A Max. (9-17Pins) |
| --- | --- |
| Rated Voltage | 30/60V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 8,17 Poles are available, 3,4,5,12 can be customized （Refer to below SKU table） |
| Water Resistance Level | IP67, IP68 in locked condition |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-M12ASTM-03-001 | 3 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-04-001 | 4 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-05-001 | 5 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-08-001 | 8 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-12-001 | 12 | Male | Straight | With Metal Nut/Screw thread |
| T-M12ASTM-17-001 | 17 | Male | Straight | With Metal Nut/Screw thread |

The Dimensions of Related Components (Unit is mm)

- General Information of Connector

Ambient Temperature:  -20℃ ~ +80℃                   Insulation Resistance:   ≥100MΩ

Connector Body:       PA66                           Contact Resistance:     ≤10mΩ

Connector Contacts:    Brass with gold plated             Shield:                Available

Coupling Nut/Screw:    Zinc alloy with nickel plated        IP Rating:              IP67,IP68 in locked condition

Mating Endurance:     >500 Cycles

M12 Female (A Coding) Molded Cable, Right Angle, With Metal Nut (Screw Thread)

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 2A Max (6-8Pins), 4A Max (2-5Pins), 1.5A Max. (9-17Pins) |
| --- | --- |
| Rated Voltage | 30/60V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 5,8 Poles are available, 3,4,12,17 can be customized （Refer to below SKU table） |
| Water Resistance Level | IP67, IP68 in locked condition |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-M12ARAF-03-001 | 3 | Female | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAF-04-001 | 4 | Female | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAF-05-001 | 5 | Female | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAF-08-001 | 8 | Female | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAF-12-001 | 12 | Female | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAF-17-001 | 17 | Female | Right Angle | With Metal Nut/Screw thread |

Dimensions of Related Components (Unit is mm)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                   Insulation Resistance:    ≥100MΩ

Connector Body:        PA66                           Contact Resistance:      ≤10mΩ

Connector Contacts:     Brass with gold plated              Shield:                Available

Coupling Nut/Screw:     Zinc alloy with nickel plated         IP Rating:              IP67,IP68 in locked condition

Seal/O-ring:            Silicone                         Mating Endurance:      >500 Cycles

M12 Male (A Coding) Molded Cable, Right Angle, With Metal Nut (Screw Thread)

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 2A Max (6-8Pins), 4A Max (2-5Pins), 1.5A Max. (9-17Pins) |
| --- | --- |
| Rated Voltage | 30/60V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 3,4,5,8,12,17 can be customized （Refer to below SKU table） |
| Water Resistance Level | IP67, IP68 in locked condition |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-M12ARAM-03-001 | 3 | Male | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAM-04-001 | 4 | Male | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAM-05-001 | 5 | Male | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAM-08-001 | 8 | Male | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAM-12-001 | 12 | Male | Right Angle | With Metal Nut/Screw thread |
| T-M12ARAM-17-001 | 17 | Male | Right Angle | With Metal Nut/Screw thread |

Dimensions of Related Components (Unit is mm)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PA66                           Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with gold plated             Shield:                Available

Coupling Nut/Screw:     Zinc alloy with nickel plated        IP Rating:             IP67,IP68 in locked condition

Mating Endurance:      >500 Cycles

M12 Male (D Coding) Molded Cable, Straight, With Metal Nut (Screw Thread)

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V/60V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4 Poles straight are available, right angle can be customized (refer to SKU Table) |
| Water Resistance Level | IP67 in locked condition, IP68 Can be customized |
| Cable | PUR/PVC cable (18-24AWG) options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-M12DSTM-04-001 | 4 | Male | Straight | With Metal Nut/Screw thread |
| T-M12DRAM-04-001 | 4 | Male | Right angle | With Metal Nut/Screw thread |

- Dimensions of Related Components (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +80℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PA66                           Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with gold plated              Shield:               Available

Coupling Nut/Screw:    Zinc alloy with nickel plated          IP Rating:             IP67 in locked condition

Mating Endurance:     >500 Cycles

M12 Female (D Coding) Molded Cable, Straight, With Metal Nut (Screw Thread)

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-M12DSTF-04-001 | 4 | Female | Straight | With Metal Nut/Screw thread |
| T-M12DRAF-04-001 | 4 | Female | Right angle | With Metal Nut/Screw thread |

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V/60V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4 Poles straight are available, right angle can be customized (refer to SKU Table) |
| Water Resistance Level | IP67 in locked condition, IP68 Can be customized |
| Cable | PUR/PVC cable (18-24AWG) options |

- Dimensions of Related Components (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +80℃                  Insulation Resistance:    ≥100MΩ

Connector Body:        PA66                          Contact Resistance:      ≤10mΩ

Connector Contacts:     Brass with gold plated             Shield:                Available

Coupling Nut/Screw:     Zinc alloy with nickel plated        IP Rating:              IP67 in locked condition

Mating Endurance:      >500 Cycles

| M12 Male (X Coding) Molded Cable, Straight, With Metal Nut (Screw Thread) - Dimensions of Cable Assembly (Unit is mm) - General Information of Cable Assembly Rated Current; 0.5A Max.. Rated Voltage; 48V DC. Insulation Resistance; ≥100MΩ. Contact Resistance; ≤5Ω. Operation Temperature; -25℃ ~+85℃. Poles; 8 Poles straight are available, right angle can be customized (refer to SKU Table). Water Resistance Level; IP67 in locked condition, IP68 Can be customized. Cable; PUR/PVC cable (24, 26AWG) options. - SKU Table SKU No.; Poles; Conn. Type; Overmoding Type; Remark. T-M12XSTM-08-001; 8; Male; Straight; With Metal Nut/Screw thread. T-M12XRAM-08-001; 8; Male; Right angle; With Metal Nut/Screw thread. - Dimensions of Related Components (Unit is mm) - General Information of Connector Ambient Temperature: -25℃ ~ +85℃ Insulation Resistance: ≥100MΩ Connector Body: PA66 Contact Resistance: ≤5mΩ Connector Contacts: Brass with gold plated Shield: Available Coupling Nut/Screw: Zinc alloy with nickel plated IP Rating: IP67 in locked condition Mating Endurance: >500 Cycles M8 Female (A Coding) Molded Cable, Straight, With Metal Nut (Screw Thread) - Dimensions of Cable Assembly (Unit is mm) - General Information of Cable Assembly Rated Current; 1.5A Max (6-8Pins), 3A Max (3-5Pins). Rated Voltage; 30/60V DC. Insulation Resistance; ≥100MΩ. Contact Resistance; ≤5Ω. Operation Temperature; -20℃ ~+80℃. Poles; 3,4,6 Poles straight are available, 5,8 straight can be customized （Refer to below SKU table） 3,4,5,6,8 right angle can be customized too （Refer to below SKU table）. Water Resistance Level; IP67, IP68 in locked condition. Cable; PUR/PVC cable (24-26AWG) options. - Pins Layout - SKU Table SKU No.; Poles; Conn. Type; Overmoding Type; Remark. T-M8ASTF-03-001; 3; Female; Straight; With Metal Nut/Screw thread. T-M8ASTF-04-001; 4; Female; Straight; With Metal Nut/Screw thread. T-M8ASTF-05-001; 5; Female; Straight; With Metal Nut/Screw thread. T-M8ASTF-06-001; 6; Female; Straight; With Metal Nut/Screw thread. T-M8ASTF-08-001; 8; Female; Straight; With Metal Nut/Screw thread. T-M8ARAF-03-001; 3; Female; Right Angle; With Metal Nut/Screw thread. T-M8ARAF-04-001; 4; Female; Right Angle; With Metal Nut/Screw thread. T-M8ARAF-05-001; 5; Female; Right Angle; With Metal Nut/Screw thread. T-M8ARAF-06-001; 6; Female; Right Angle; With Metal Nut/Screw thread. T-M8ARAF-08-001; 8; Female; Right Angle; With Metal Nut/Screw thread. - Dimensions of Related Components (Unit is mm) - General Information of Connector Ambient Temperature: -25℃ ~ +85℃ Insulation Resistance: ≥100MΩ Connector Body: PA66 Contact Resistance: ≤5mΩ Connector Contacts: Brass with gold plated Shield: Available Coupling Nut/Screw: Zinc alloy with nickel plated IP Rating: IP67 in locked condition Mating Endurance: >500 Cycles M8 Male (A Coding) Molded Cable, Straight, With Metal Nut (Screw Thread) - General Information of Cable Assembly Rated Current; 1.5A Max (6-8Pins), 3A Max (3-5Pins). Rated Voltage; 30/60V DC. Insulation Resistance; ≥100MΩ. Contact Resistance; ≤5Ω. Operation Temperature; -20℃ ~+80℃. Poles; 3,4,6 Poles straight are available, 5,8 straight can be customized （Refer to below SKU table） 3,4,5,6,8 right angle can be customized too （Refer to below SKU table）. Water Resistance Level; IP67 in locked condition. Cable; PUR/PVC cable (24-26AWG) options. - SKU Table SKU No.; Poles; Conn. Type; Overmoding Type; Remark. T-M8ASTM-03-001; 3; Male; Straight; With Metal Nut/Screw thread. T-M8ASTM-04-001; 4; Male; Straight; With Metal Nut/Screw thread. T-M8ASTM-05-001; 5; Male; Straight; With Metal Nut/Screw thread. T-M8ASTM-06-001; 6; Male; Straight; With Metal Nut/Screw thread. T-M8ASTM-08-001; 8; Male; Straight; With Metal Nut/Screw thread. T-M8ARAM-03-001; 3; Male; Right Angle; With Metal Nut/Screw thread. T-M8ARAM-04-001; 4; Male; Right Angle; With Metal Nut/Screw thread. T-M8ARAM-05-001; 5; Male; Right Angle; With Metal Nut/Screw thread. T-M8ARAM-06-001; 6; Male; Right Angle; With Metal Nut/Screw thread. T-M8ARAM-08-001; 8; Male; Right Angle; With Metal Nut/Screw thread. - Pins Layout - Dimensions of Cable Assembly (Unit is mm) - Dimensions of Components |  |  |
| --- | --- | --- |
| - General Information of Connector Ambient Temperature: -25℃ ~ +85℃ Insulation Resistance: ≥100MΩ Connector Body: PA66 Contact Resistance: ≤5mΩ Connector Contacts: Brass with gold plated Shield: Available Coupling Nut/Screw: Zinc alloy with nickel plated IP Rating: IP67 in locked condition Mating Endurance: >500 Cycles |  |  |

DIN Circle Female Molded Cable, Straight, With Shell

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 2A Max. |
| --- | --- |
| Rated Voltage | 12V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 8 Poles straight are available, 3-8 Poles straight can be customized （Refer to below SKU table） |
| Design Standard | Per VDE DIN standard |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DINSTF-03-001 | 3 | Female | Straight | With shell |
| T- DINSTF-04-001 | 4 | Female | Straight | With shell |
| T- DINSTF-05-001 | 5 | Female | Straight | With shell, Pins 180 degree |
| T- DINSTF-05-002 | 5 | Female | Straight | With shell, one pin in the middle |
| T- DINSTF-06-001 | 6 | Female | Straight | With shell |
| T- DINSTF-07-001 | 7 | Female | Straight | With shell |
| T- DINSTF-08-001 | 8 | Female | Straight | With shell |

- Dimensions of Related Components (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT+GF/ABS                     Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with silver/tin               Shell:                 Steel with nickel          Mating Endurance:      5000 Cycles                      Pull Force:             3.5+/1 kg

DIN Circle Male Molded Cable, Straight, With Shell

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 2A Max. |
| --- | --- |
| Rated Voltage | 12V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 8 Poles straight are available, 3-8 Poles straight can be customized （Refer to below SKU table） |
| Design Standard | Per VDE DIN standard |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DINSTM-03-001 | 3 | Male | Straight | With shell |
| T- DINSTM-04-001 | 4 | Male | Straight | With shell |
| T- DINSTM-05-001 | 5 | Male | Straight | With shell, Pins with 180 degree |
| T- DINSTM-05-002 | 5 | Male | Straight | With shell, Pins within 240 degree |
| T- DINSTM-06-001 | 6 | Male | Straight | With shell |
| T- DINSTM-07-001 | 7 | Male | Straight | With shell |
| T- DINSTM-08-001 | 8 | Male | Straight | With shell |

- Dimensions of Related Components (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT+GF/ABS                     Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with silver/tin               Shell:                 Steel with nickel           Mating Endurance:      5000 Cycles                      Pull Force:             3.5+/1 kg

DIN Male Molded Cable, Right Angle, With Shell

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 2A Max. |
| --- | --- |
| Rated Voltage | 12V DC |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 8 Poles straight are available, 3-8 Poles straight can be customized （Refer to below SKU table） |
| Design Standard | Per VDE DIN standard |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DINRAM-03-001 | 3 | Male | Right Angle | With shell |
| T- DINRAM-04-001 | 4 | Male | Right Angle | With shell |
| T- DINRAM-05-001 | 5 | Male | Right Angle | With shell, Pins with 180 degree |
| T- DINRAM-05-002 | 5 | Male | Right Angle | With shell, Pins within 240 degree |
| T- DINRAM-06-001 | 6 | Male | Right Angle | With shell |
| T- DINRAM-07-001 | 7 | Male | Right Angle | With shell |
| T- DINRAM-08-001 | 8 | Male | Right Angle | With shell |

- Dimensions of Related Components (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +85℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT+GF/ABS                     Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with silver/tin               Shell:                 Steel with nickel           Mating Endurance:      5000 Cycles                      Pull Force:             3.5+/1 kg

9Pin MINI DIN Female Molded Cable, Straight, With Shell

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 1.5A Max. |
| --- | --- |
| Rated Voltage | 12V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 3-9 Poles straight can be customized（Refer to below SKU table） |
| Design Standard | Per VDE DIN standard |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-MDINSTF-03-001 | 3 | Female | Straight | With shell |
| T- MDINSTF-04-001 | 4 | Female | Straight | With shell |
| T- MDINSTF-05-001 | 5 | Female | Straight | With shell |
| T- MDINSTF-06-001 | 6 | Female | Straight | With shell |
| T- MDINSTF-07-001 | 7 | Female | Straight | With shell |
| T- MDINSTF-08-001 | 8 | Female | Straight | With shell |
| T- MDINSTF-09-001 | 9 | Female | Straight | With shell |

- Dimensions of Related Components (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT+GF/ABS                     Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with silver/tin               Shell:                 Steel with nickel

MINI DIN Male Molded Cable, Straight, With Shell

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 1.5A Max. |
| --- | --- |
| Rated Voltage | 12V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 6, 8 Poles are available, 3-9 Poles straight can be customized（Refer to SKU table） |
| Design Standard | Per VDE DIN standard |
| Cable | PUR/PVC cable (22-28AWG) options |

- Pins Layout

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-MDINSTM-03-001 | 3 | Male | Straight | With shell |
| T- MDINSTM-04-001 | 4 | Male | Straight | With shell |
| T- MDINSTM-05-001 | 5 | Male | Straight | With shell |
| T- MDINSTM-06-001 | 6 | Male | Straight | With shell |
| T- MDINSTM-07-001 | 7 | Male | Straight | With shell |
| T- MDINSTM-08-001 | 8 | Male | Straight | With shell |
| T- MDINSTM-09-001 | 9 | Male | Straight | With shell |

- Dimensions of Related Components (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT+GF/ABS                     Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with silver/tin               Shell:                 Steel with nickel

9Pin D-SUB Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly  (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 9 Poles straight are available, 9 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTF-09-001 | 9 | Female | Straight | Locking type: Fix screw |
| T- DSUBRAF-09-001 | 9 | Female | Right Angle | Locking type: Fix screw |

- Dimensions of Related Components (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                 Unavailable

Pitch：                2.84mm                        Mating Endurance:      500 Cycles

9 Pin D-SUB Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly  (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTM-09-001 | 9 | Male | Straight | Locking type: Fix screw |
| T- DSUBRAM-09-001 | 9 | Male | Right Angle | Locking type: Fix screw |

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 9 Poles straight are available, 9 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

Connector

Dimensions of Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                 Unavailable

Pitch：                2.84mm                        Mating Endurance:      500 Cycles

15 Pin D-SUB Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTF-15-001 | 15 | Female | Straight | Locking type: Fix screw |
| T- DSUBRAF-15-001 | 15 | Female | Right Angle | Locking type: Fix screw |

- General Information of Cable Assembly (Unit is mm)

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 15 Poles straight are available, 15 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                 Unavailable

Pitch：                2.84mm                        Mating Endurance:      500 Cycles

15 Pin D-SUB Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 15 Poles straight are available, 15 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTM-15-001 | 15 | Male | Straight | Locking type: Fix screw |
| T- DSUBRAM-15-001 | 15 | Male | Right Angle | Locking type: Fix screw |

Connector

Dimensions of Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                 Unavailable

Pitch：                2.84mm                        Mating Endurance:      500 Cycles

25 Pin D-SUB Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTF-25-001 | 25 | Female | Straight | Locking type: Fix screw |
| T- DSUBRAF-25-001 | 25 | Female | Right Angle | Locking type: Fix screw |

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 25 Poles straight are available, 25 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                 Unavailable

Pitch：                2.84mm                        Mating Endurance:      500 Cycles

25 Pin D-SUB Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 25 Poles straight are available, 25 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTM-25-001 | 25 | Male | Straight | Locking type: Fix screw |
| T- DSUBRAM-25-001 | 25 | Male | Right Angle | Locking type: Fix screw |

Connector

Dimensions of Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                 Unavailable

Pitch：                2.84mm                        Mating Endurance:      500 Cycles

37Pin D-SUB Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly  (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 37 Poles straight and right angle can be customized (Refer to below SKU table) |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTF-37-001 | 37 | Female | Straight | Locking type: Fix screw |
| T- DSUBRAF-37-001 | 37 | Female | Right Angle | Locking type: Fix screw |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                Unavailable

Pitch：                2.84mm                        Mating Endurance:     500 Cycles

37 Pin D-SUB Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-DSUBSTM-37-001 | 37 | Male | Straight | Locking type: Fix screw |
| T- DSUBRAM-37-001 | 37 | Male | Right Angle | Locking type: Fix screw |

- General Information of Cable Assembly

| Rated Current | 4A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 37 Poles straight are available, 37 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

Connector

Dimensions of Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                           Contact Resistance:     ≤10mΩ

Connector Contacts:     Brass with nickel/gold plated       Shell:                Unavailable

Pitch：                2.84mm                       Mating Endurance:      500 Cycles

15Pin High Density D-SUB Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-HDBSTF-15-001 | 15 | Female | Straight | Locking type: Fix screw |
| T-HDBSTF-26-001 | 26 | Female | Straight | Locking type: Fix screw |
| T-HDBSTF-44-001 | 44 | Female | Straight | Locking type: Fix screw |
| T-HDBSTF-62-001 | 62 | Female | Straight | Locking type: Fix screw |
| T-HDBSTF-78-001 | 78 | Female | Straight | Locking type: Fix screw |
| T-HDBRAF-15-001 | 15 | Female | Right Angle | Locking type: Fix screw |
| T-HDBRAF-26-001 | 26 | Female | Right Angle | Locking type: Fix screw |
| T-HDBRAF-44-001 | 44 | Female | Right Angle | Locking type: Fix screw |
| T-HDBRAF-62-001 | 62 | Female | Right Angle | Locking type: Fix screw |
| T-HDBRAF-78-001 | 78 | Female | Right Angle | Locking type: Fix screw |

- General Information of Cable Assembly

| Rated Current | 5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 15 Poles straight is available, 26, 44,62,78 Poles straight/right angle can be customized (Refer to SKU table) |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

Connector

Dimensions of Connector  (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤15mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                Unavailable

Pitch：                1.98mm                        Mating Endurance:     500 Cycles

15Pin High Density D-SUB Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-HDBSTM-15-001 | 15 | Male | Straight | Locking type: Fix screw |
| T-HDBSTM-26-001 | 26 | Male | Straight | Locking type: Fix screw |
| T-HDBSTM-44-001 | 44 | Male | Straight | Locking type: Fix screw |
| T-HDBSTM-62-001 | 62 | Male | Straight | Locking type: Fix screw |
| T-HDBSTM-78-001 | 78 | Male | Straight | Locking type: Fix screw |
| T- HDBRAM-15-001 | 15 | Male | Right Angle | Locking type: Fix screw |
| T- HDBRAM-26-001 | 26 | Male | Right Angle | Locking type: Fix screw |
| T- HDBRAM-44-001 | 44 | Male | Right Angle | Locking type: Fix screw |
| T- HDBRAM-62-001 | 62 | Male | Right Angle | Locking type: Fix screw |
| T- HDBRAM-78-001 | 78 | Male | Right Angle | Locking type: Fix screw |

- General Information of Cable Assembly

| Rated Current | 5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+70℃ |
| Poles | 15 Poles straight are available, 26, 44,62,78 Poles straight/right angle can be customized （Refer to SKU table） |
| Cable | PUR/PVC cable (18-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                           Contact Resistance:     ≤15mΩ

Connector Contacts:     Brass with nickel/gold plated       Shell:                Unavailable

Pitch：                1.98mm                       Mating Endurance:     500 Cycles

27W2 Hybrid D-SUB Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 7.5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 5W5,7W2, 9W4, 11W1, 13W3, 27W2 Poles straight/right angle can be customized (Refer to SKU table) |
| Cable | PUR/PVC cable (16-20AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

Connector

Dimensions of Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -25℃ ~ +70℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤15mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                Unavailable

Pitch：                2.84mm                        Mating Endurance:     500 Cycles.

27W2 Hybrid D-SUB Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 7.5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 27W2 Poles straight are available, 5W5,7W2, 9W4, 11W1, 13W3 Poles straight/right angle can be customized (Refer to SKU table) |
| Cable | PUR/PVC cable (16-20AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-HDSUBSTM-05-001 | 5+0 | Male | Straight | Locking type: Fix screw |
| T-HDSUBSTM-07-001 | 5+2 | Male | Straight | Locking type: Fix screw |
| T-HDSUBSTM-09-001 | 5+4 | Male | Straight | Locking type: Fix screw |
| T-HDSUBSTM-11-001 | 10+1 | Male | Straight | Locking type: Fix screw |
| T-HDSUBSTM-13-001 | 10+3 | Male | Straight | Locking type: Fix screw |
| T-HDSUBSTM-27-001 | 25+2 | Male | Straight | Locking type: Fix screw |
| T-HDSUBRAM-05-001 | 5+0 | Male | Right Angle | Locking type: Fix screw |
| T-HDSUBRAM-07-001 | 5+2 | Male | Right Angle | Locking type: Fix screw |
| T-HDSUBRAM-09-001 | 5+4 | Male | Right Angle | Locking type: Fix screw |
| T-HDSUBRAM-11-001 | 10+1 | Male | Right Angle | Locking type: Fix screw |
| T-HDSUBRAM-13-001 | 10+3 | Male | Right Angle | Locking type: Fix screw |
| T-HDSUBRAM-27-001 | 25+2 | Male | Right Angle | Locking type: Fix screw |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -35℃ ~ +85℃                   Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                            Contact Resistance:     ≤15mΩ

Connector Contacts:     Brass with nickel/gold plated        Shell:                Available

Pitch：                2.84mm                        Mating Endurance:     500 Cycles

9Pin Micro D-SUB Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-MDSUBSTF-09-001 | 9 | Female | Straight |  |
| T-MDSUBRAF-09-001 | 9 | Female | Right Angle |  |
| T-MDSUBSTF-15-001 | 15 | Female | Straight |  |
| T-MDSUBRAF-15-001 | 15 | Female | Right Angle |  |
| T-MDSUBSTF-25-001 | 25 | Female | Straight |  |
| T-MDSUBRAF-25-001 | 25 | Female | Right Angle |  |

- General Information of Cable Assembly

| Rated Current | 1A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 9, 15, 25 Poles straight/right angle can be customized (Refer to below SKU table) |
| Cable | PUR/PVC cable (26-28AWG) options |
| Water Resistance | IP67 in lock condition can be customized if need |

Connector

Dimensions of Connector (Unit is mm)

- General Information of Connector

Ambient Temperature:   -35℃ ~ +85℃                  Insulation Resistance:   ≥100MΩ

Connector Body:        PBT                           Contact Resistance:     45mΩ max.

Connector Contacts:     Brass with nickel/gold plated       Shell:                 Available

Pitch：                1.27mm                       Mating Endurance:      500 Cycles

USB 2.0 Type A Female Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0ASTF-04-001 | 4 | Female | Straight |  |

- General Information of Cable Assembly

| Rated Current | 1.5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4 Poles straight is available, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable ,PUR/PVC options |

Connector

Related Components (Unit is mm)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                     Insulation Resistance:    ≥100MΩ

Connector Body:        PBT, BLK/WHT                      Contact Resistance:     30mΩ Max.

Connector Contacts:     Copper Alloy with gold plated          Shell:                 Stainless steel

Fire Resistance：       UL94V-0                           Insert Force:           35N max.

USB 2.0 Type A Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 1.5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4 Poles straight is available, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable ,PUR/PVC options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0ASTM-04-001 | 4 | Male | Straight |  |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                      Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, BLK/WHT                       Contact Resistance:     30mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated       Shell:                Stainless steel

Fire Resistance：       UL94V-0                            Insert Force:           35N max.

USB 2.0 Type A Male Molded Cable Assembly, Right Angle

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 1.5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4 Poles is available, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable, PUR/PVC options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0ARAM-04-001 | 4 | Male | Right Angle |  |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                     Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, BLK/WHT                      Contact Resistance:     30mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated       Shell:                Stainless steel

Fire Resistance：       UL94V-0                           Insert Force:           35N max.

USB 2.0 Type B Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 1.5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4 Poles straight is available, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable, PUR/PVC options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0BSTM-04-001 | 4 | Male | Straight |  |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                      Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, BLK/WHT                       Contact Resistance:     30mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated       Shell:                 Stainless steel

Fire Resistance：        UL94V-0                           Insert Force:           35N max.

USB 2.0 Type B Male Molded Cable Assembly, Right Angle

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 1.5A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 4 Poles is available, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable, PUR/PVC options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0BRAM-04-001 | 4 | Male | Right angle |  |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                     Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, BLK/WHT                      Contact Resistance:     30mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated      Shell:                 Stainless steel

Fire Resistance：       UL94V-0                           Insert Force:           35N max.

USB 2.0 Mini-B Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | 1.0 A Max. |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+60℃ |
| Poles | 5 Poles straight is available, right angle can be customized, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable, PUR/PVC options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0MINBSTM-05-001 | 5 | Male | Straight |  |
| T-USB2.0MINBRAM-05-001 | 5 | Male | Right Angle |  |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +60℃                      Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, BLK/WHT                       Contact Resistance:     30mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated       Shell:                 Stainless steel

Fire Resistance：       UL94V-0                            Insert Force:           35N max.

USB 2.0 Micro B Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | Signal (Pins 2,3,4): 1.0A Max., Power (Pins 1,5): 1.8 A |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 5 Poles is available, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable, PUR/PVC options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0MICBSTM-05-001 | 5 | Male | Straight |  |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                      Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, BLK/WHT                      Contact Resistance:     50mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated       Shell:                Stainless steel

Fire resistance：        UL94V-0                           Insert Force:           35N max.

USB 2.0 Micro B Male Molded Cable Assembly, right angle

- Dimensions of Cable Assembly (Unit is mm)

- General Information of Cable Assembly

| Rated Current | Signal (Pins 2,3,4): 1.0A Max., Power (Pins 1,5): 1.8 A |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+50℃ |
| Poles | 5 Poles is available, refer to below SKU table |
| Cable | USB2.0 24-28AWG cable ,PUR/PVC options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB2.0MICBRAM-05-001 | 5 | Male | Right angle |  |

- Dimensions of Connector (Unit is mm)

Connector

- General Information of Connector

Ambient Temperature:   -20℃ ~+80℃                      Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, BLK/WHT                      Contact Resistance:     50mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated       Shell:                Stainless steel

Fire resistance：        UL94V-0                           Insert Force:           35N max.

USB 3.0 Type A Female Molded Cable Assembly

- Dimensions of Cable Assembly (Unit is mm)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB3.0ARAF-09-001 | 9 | Female | Right Angle |  |
| T-USB3.0ASTF-09-001 | 9 | Female | Straight |  |

- General Information of Cable Assembly

| Transmission Speed | 5Gbps(640MB/s) |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 9 Poles right angle is available, straight can be customized ( Refer to SKU table） |
| Cable | PUR/PVC cable (28AWG) options |

Connector

Related Components (Unit is mm, refer to if it is without tolerance)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                     Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, Blue                          Contact Resistance:     50mΩ Max.

Connector Contacts:     Copper Alloy with gold plated         Shell:                 Stainless steel

Fire resistance：        UL94V-0                          Mating Force:          3.57KG max.

USB 3.0 Type A Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm, refer to if it is without tolerance)

- General Information of Cable Assembly

| Transmission Speed | 5Gbps(640MB/s) |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥100MΩ |
| Contact Resistance | ≤5Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 9 Poles straight are available, 9 Poles right angle can be customized （Refer to below SKU table） |
| Cable | PUR/PVC cable (28AWG) options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB3.0ASTM-09-001 | 9 | Male | Straight |  |
| T-USB3.0ARAM-09-001 | 9 | Male | Right Angle |  |

Connector

Dimensions of Connector (Unit is mm, refer to if it is without tolerance)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                     Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, blue                          Contact Resistance:     50mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated      Shell:                 Available

Fire resistance：        UL94V-0                          Mating Force:          3.57KG max.

USB 3.0 Micro-B Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (mm, refer to if it is without tolerance)

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB3.0MICBSTM-10-001 | 10 | Male | Straight |  |

- General Information of Cable Assembly

| Transmission Speed | 5Gbps(640MB/s) |
| --- | --- |
| Rated Voltage | 30V |
| Insulation Resistance | ≥10MΩ |
| Contact Resistance | ≤3Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | 10 Poles straight are available, Refer to below SKU table |
| Cable | PUR/PVC cable (28AWG) options |

Dimensions of Connector (Unit is mm, refer to if it is without tolerance)

- General Information of Connector

Ambient Temperature:   -20℃ ~ +80℃                     Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, black                         Contact Resistance:     50mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated      Shell:                 Available

Fire resistance：        UL94V-0                          Mating Force:          3.57KG max.

Type C Male Molded Cable Assembly, Straight

- Dimensions of Cable Assembly (Unit is mm, refer to if it is without tolerance)

- General Information of Cable Assembly

| Transmission Speed | 10Gbit (1000MB/s) |
| --- | --- |
| Rated Voltage | 30V |
| Rated Current/Power delivery | 5A Max. /Also provide the 100W (20V/5A) electric power |
| Insulation Resistance | ≥10MΩ |
| Contact Resistance | ≤3Ω |
| Operation Temperature | -20℃ ~+80℃ |
| Poles | A12+B12 Poles straight and right angle are available, Refer to below SKU table |
| Cable | PUR/PVC cable (28-32AWG) options |

- SKU Table

| SKU No. | Poles | Conn. Type | Overmoding Type | Remark |
| --- | --- | --- | --- | --- |
| T-USB3.0TCSTM-24-001 | 24 | Male | Straight |  |
| T-USB3.0TCRAM-24-001 | 24 | Male | Right angle |  |

Pinout definition of male

Dimensions of Connector (Unit is mm, refer to if it is without tolerance)

- General Information of Connector

Ambient Temperature:   -55℃ ~ +85℃                     Insulation Resistance:   ≥100MΩ

Connector Body:        PBT, black                         Contact Resistance:     30mΩ Max.

Connector Contacts:     Phosphor Bronze with gold plated      Life Cycles:            10K Times

Fire resistance：        UL94V-0                          Mating/Pull Force:      3.57Kgf max./1.02Kgf Min.